Computer Networks Lab 08

Syed Huzaifa Ali

23K-0004

BAI-6A

Code:

#include <iostream>

#include <fstream>

#include <string>

#include "ns3/core-module.h"

#include "ns3/applications-module.h"

#include "ns3/network-module.h"

#include "ns3/internet-module.h"

#include "ns3/point-to-point-module.h"

#include "ns3/ipv4-global-routing-helper.h"

using namespace ns3;

NS_LOG_COMPONENT_DEFINE ("TcpLargeTransfer");

static const uint32_t totalTxBytes = 2000000;

static uint32_t currentTxBytes = 0;

static const uint32_t writeSize = 1040;

uint8_t data[writeSize];

void StartFlow (Ptr<Socket>, Ipv4Address, uint16_t);

void WriteUntilBufferFull (Ptr<Socket>, uint32_t);

static std::ofstream cwndFile;

static void

CwndTracer (uint32_t oldval, uint32_t newval)

{

NS_LOG_INFO ("Moving cwnd from " << oldval << " to " << newval);

cwndFile << Simulator::Now ().GetSeconds () << "\t" << newval << "\n";

}

int main (int argc, char *argv[])

{

cwndFile.open ("cwnd-output.dat");

CommandLine cmd;

cmd.Parse (argc, argv);

for (uint32_t i = 0; i < writeSize; ++i)

{

char m = toascii (97 + i % 26);

data[i] = m;

}

NodeContainer n0n1;

n0n1.Create (2);

NodeContainer n1n2;

n1n2.Add (n0n1.Get (1));

n1n2.Create (1);

PointToPointHelper p2p;

p2p.SetDeviceAttribute ("DataRate", DataRateValue (DataRate (10000000)));

p2p.SetChannelAttribute ("Delay", TimeValue (MilliSeconds (10)));

NetDeviceContainer dev0 = p2p.Install (n0n1);

NetDeviceContainer dev1 = p2p.Install (n1n2);

InternetStackHelper internet;

internet.InstallAll ();

Ipv4AddressHelper ipv4;

ipv4.SetBase ("10.1.3.0", "255.255.255.0");

ipv4.Assign (dev0);

ipv4.SetBase ("10.1.2.0", "255.255.255.0");

Ipv4InterfaceContainer ipInterfs = ipv4.Assign (dev1);

Ipv4GlobalRoutingHelper::PopulateRoutingTables ();

uint16_t servPort = 50000;

PacketSinkHelper sink ("ns3::TcpSocketFactory",

InetSocketAddress (Ipv4Address::GetAny (), servPort));

ApplicationContainer apps = sink.Install (n1n2.Get (1));

apps.Start (Seconds (0.0));

apps.Stop (Seconds (3.0));

Ptr<Socket> localSocket =

Socket::CreateSocket (n0n1.Get (0), TcpSocketFactory::GetTypeId ());

localSocket->Bind ();

Config::ConnectWithoutContext (

"/NodeList/0/$ns3::TcpL4Protocol/SocketList/0/CongestionWindow",

MakeCallback (&CwndTracer));

Simulator::ScheduleNow (&StartFlow, localSocket,

ipInterfs.GetAddress (1), servPort);

AsciiTraceHelper ascii;

p2p.EnableAsciiAll (ascii.CreateFileStream ("tcp-large-transfer.tr"));

p2p.EnablePcapAll ("tcp-large-transfer");

Simulator::Stop (Seconds (1000));

Simulator::Run ();

Simulator::Destroy ();

cwndFile.close ();

return 0;

}

void StartFlow (Ptr<Socket> localSocket,

Ipv4Address servAddress,

uint16_t servPort)

{

NS_LOG_LOGIC ("Starting flow at time " << Simulator::Now ().GetSeconds ());

localSocket->Connect (InetSocketAddress (servAddress, servPort));

localSocket->SetSendCallback (MakeCallback (&WriteUntilBufferFull));

WriteUntilBufferFull (localSocket, localSocket->GetTxAvailable ());

}

void WriteUntilBufferFull (Ptr<Socket> localSocket, uint32_t txSpace)

{

while (currentTxBytes < totalTxBytes && localSocket->GetTxAvailable () > 0)

{

uint32_t left = totalTxBytes - currentTxBytes;

uint32_t dataOffset = currentTxBytes % writeSize;

uint32_t toWrite = writeSize - dataOffset;

toWrite = std::min (toWrite, left);

toWrite = std::min (toWrite, localSocket->GetTxAvailable ());

int amountSent = localSocket->Send (&data[dataOffset], toWrite, 0);

if (amountSent < 0)

{

return;

}

currentTxBytes += amountSent;

}

localSocket->Close ();

}

Graph: